[image: image1.png]


BMW
Media
Information

5/2010

valid from 5/2010

BMW
Media
Information

5/2010

valid from 5/2010


Technical Specifications BMW M3 GTS   (05/2010)
(
	
	
	BMW M3 GTS 
	
	

	
	
	
	
	

	Body
	
	
	
	

	Number of doors/seats
	
	2 / 2
	
	

	Length/Width/Height (unladen)
	mm
	4645 / 1804 / 1387
	
	

	Wheelbase
	mm
	2760
	
	

	Track front/rear
	mm
	1546 / 1539
	
	

	Turning cycle
	m
	11.9
	
	

	Tank capacity
	appr ltr
	63
	
	

	Cooling system incl. heating
	ltr
	11-4
	
	

	Engine oil3)
	ltr
	8.8
	
	

	Weight, unladen, to DIN/EU
	kg
	1530 / 1605
	
	


	Max load to DIN
	kg
	350


	
	

	Max perm weight, overall
	kg
	1880
	
	

	Max perm axle load front/rear
	kg
	950 / 980
	
	

	Max perm trailer load (12 %)
braked/unbraked
	kg
	– / –
	
	

	Perm roof load/trail download
	kg
	– / –
	
	

	Luggage capacity
	l
	430
	
	

	Air resistance
	cd x A
	0.34 x 2.17
	
	

	
	
	
	
	

	Engine
	
	
	
	

	Config/no of cyls/valves
	
	V / 8 / 4
	
	

	Engine technology
	 
	High-revving concept with individual throttle valves, 
Bi-Vanos, dynamically optimised oil supply and 
ionic-current knock control
	
	

	Capacity
	cc
	4361
	
	

	Stroke/bore
	mm
	82.0 / 92.0
	
	

	Compression ratio
	:1
	12.0 
	
	

	Fuel grade
	
	at least RON 95
	
	

	Max output
	kW/bhp
	331 / 450
	
	

	at
	min–1
	8300
	
	

	Max torque
	Nm
	440
	
	

	at
	min–1
	3750
	
	

	
	
	
	
	

	Electrical system
	
	
	
	

	Battery/installation
	Ah/–
	70 / luggage comp
	
	

	Alternator
	A/W
	180 / 2520
	
	

	
	
	
	
	

	Driving dynamics and safety
	

	Suspension, front
	
	Aluminium double-joint spring-strut suspension with tension rod; small, positive steering scrub radius; traverse force compensation; anti-dive, coilover suspension with dampers independently variable in compression and rebound travel, variable wheel camber

	Suspension, rear
	
	Lightweight five-link rear suspension with anti-squat and anti-dive, rigid rear-axle support, coilover suspension with dampers independently variable in compression and rebound travel, variable wheel camber

	Brakes, front
	
	Six-piston fixed-calliper compound disc brakes

	Diameter
	mm
	378 x 32, vented and perforated

	Brakes, rear
	
	Four-piston fixed-calliper compound disc brakes

	Diameter
	mm
	380 x 28, vented and perforated

	Driving stability systems
	  
	Fitted as standard: DSC incl. ABS and DTC (Dynamic Traction Control), CBC Cornering Brake Control, DBC Dynamic Brake Control, rain brake support, 
fading compensation, hill-start assistant, variable M differential lock, Electronic Damper Control

	Safety features
	  
	Fitted as standard: Airbags for driver and front seat passenger, side airbags for driver and front seat passenger, head airbags, three-point automatic belts with belt stoppers, pretensioners and belt-force limiters, crash sensors, 
tyre defect indicator, firmly bolted roll bar with fastening points for optional roll cage and additionally supplied six-point safety belts for driver and front seat passenger

	Steering
	
	Rack-and-pinion hydraulic steering
with Servotronic function

	Overall steering transmission
	:1
	12.5
	
	

	Tyres front/rear
	
	255/35 ZR19 /
285/30 ZR19
	
	

	Wheels front/rear
	
	9J x 19 LM /
10J x 19 LM
	
	


Technical Specifications BMW M3 GTS.

	
	
	BMW M3 GTS 
	
	

	
	
	
	
	

	Transmission
	
	
	
	

	Type
	
	7-speed M Dual Clutch Drivelogic transmission

	Gear ratios
I
	:1
	4.780
	
	

	                                  
II
	:1
	3.056
	
	

	                                  
III
	:1
	2.153
	
	

	                                  
IV
	:1
	1.678
	
	

	                                  
V
	:1
	1.390
	
	

	                                   
VI
	:1
	1.203
	
	

	                                   
VII
	:1
	1.000
	
	

	                                   
R
	:1
	4.454
	
	

	Final drive
	:1
	3.154
	
	

	
	
	
	
	

	Performance
	
	
	
	

	Power-to-weight ratio
	kg/kW
	4.6
	
	

	Output per litre
	kW/ltr
	75.9
	
	

	Acceleration
0–100 km/h
	s
	4.4
	
	

	
0–1000 m
	s
	22.5
	
	

	Top speed
	km/h
	305 
	
	

	
	
	
	
	

	Fuel con in the EU cycle
	
	
	
	

	Urban
	ltr/100km
	18.4
	
	

	Extra-urban
	ltr/100km
	9.3
	
	

	Composite
	ltr/100km
	12.7
	 
	

	CO2
	g/km
	295
	
	

	Emission classification
	
	EU5
	
	

	
	
	
	
	

	Insurance category (German)
	
	
	
	

	KH / VK / TK
	
	
	
	


Technical data valid for ACEA markets/Part of registration-specific data only apply to Germany (weights)

1)  Oil filling quantity




















































